ACKNOWLEDGEMENT

The successful completion of this project work was made possible with the help and guidance received by me from quarters. We would like to avail this opportunity to express our sincere thanks and gratitude to all of them.

We express our sincere gratitude to Dr. Narendra Vishwanath, Principal, GAT, for providing us a technically sound environment in college.

We also take this opportunity to express our heartfelt thanks to Prof. H. S. Manjunatha Reddy, Head of the Department of Electronics and Communication, GAT, who was a constant source of inspiration.

We are humbly thankful to Mr. Tilak Kumar L. and other lecturers for their support and encouragement.

We also thank our teaching and non-teaching staffs who have helped us to steer pure path to success indirectly or directly.

MARTIN J.A

RAVI S.P

UDAY WALVEKAR

VENKATAREDDY.S

ABSTRACT

A novel approach for number plate recognition is presented based on artificial neural network principles.

The images of number plate in the image database have been processed for various image processing techniques to obtain number plate using matlab software. The fundamental processes of image processing like image acquisition and morphological processing are used to perform the number plate extraction. The individual characters are then extracted using the segmentation process in which the position of each character pixel wise in rows and columns is stored in some variable. The variables are then used to extract the character by cropping it from the main image and creating a sub image.

The sub images of characters are then trained by artificial neural networks for the process of authentication. Now the trained characters are stored as a database which is used as a reference for authenticating the number plate which appears at toll gate. The valid number plates are allowed and others are not allowed.

As Neural Network is an intelligence engine, it ensures greater accuracy and better recognition speed. The recognized characters are sent over serial communication to microcontroller for display as well as the status of authentication whether valid or not.

Contents

1.  INTRODUCTION

1.1 INTRODUCTION …………………………………………………………...6

1.2 A TYPICAL NPR SYSTEM………………………………………………....6

1.3 PROPOSED NPR SYSTEM………………………………………………....6

1.4 IMAGE PROCESSING……………………………………………………...7

1.5 NEURAL NETWORK……………………………………………………..11

1.6 MICROCONTROLLER 8051…………………………….………………12

1.7 MAX 232…………………………………………………………………..13

1.8 LCD DISPLAY………………………………………..…………………...13

1.9 ULN 2803………………………………………………………………….13

1.10 STEPPER MOTOR……………………………………………………….13

1. 11PROBLEM STATEMENT…………………………………………….…14

1.12 OBJECTIVE………………………………………………………………15

2.  Literature survey

2.1 NUMBER PLATE DETECTION APPROACHES…………………….......16

2.2 CHARACTER SEGMENTATION APPROACHES…………………….....16

2.3 CHARACTER RECOGNITION APPROACHES………………………….17

2.4 CHALLENGES OF NPR SYSTEM………………………………………...18

2.5 COMMERCIAL PRODUCTS……………….……………………...............19

3. BLOCK DIAGRAM

3.1 PROJECT BLOCK DIAGRAM……………………………………………..22

3.2 BLOCK DIAGRAM FOR MICROCONTROLLER SECTION……….........24

4. DESIGN AND IMPLEMENTATION

4.1 FLOWCHART FOR IMAGE PROCESSING SECTION……………............27

4.2 FLOWCHART FOR MICROCONTROLLER SECTION…………………...28

4.3 PROJECT IMPLEMENTATION STEPS………………………………….....31

4.4 MICROCONTROLLER DATA FLOW STEPS……………..........................39

5. RESULT

5.1 SETTINGS……………………………………………………………………..41

5.2 RESULT…………………………………………………………………….…41

5.3 CONCLUSION………………………………………………………………..42

5.4 FUTURE WORK……………………………………………………………...42

6. APPLICATION OF NPR SYSTEM................................................................43

7. BIBLIOGRAPHY……………………………………………………...44

LIST OF FIGURES AND TABLES

LIST OF FIGURES

Fig 1.1 NEURAL NETWORK TRAINING DIAGRAM........................................................12

Fig 2.1 PROBLEMETIC CAPTURED IMAGES OF LICENSE PLATE..............................19

Fig 3.1 PROJECT BLOCK DIAGRAM..................................................................................22

Fig 3.2 MICROCONTROLLER CONNECTION DIAGRAM...............................................26

Fig 4.1 FLOW CHART FOR IMAGE PROCESSING SECTION....................................26-27

Fig 4.2 FLOW CHART FOR MICROCONTROLLER SECTION...................................29-30 Fig 4.3 IMPLEMENTATION STEPS FOR IMAGE PROCESSING SECTION..............30-34

Fig 4.4 EXTRACTED OBJECTS AND RESIZED IMAGES……………………………….37

Fig 4.5 CHARACTER DISPLAY……………………………………………………………38

LIST OF TABLES

Table 5.1 PLATE LOCALIZATION.......................................................................................42

Table 5.2 CHARACTER SEGMENTATION.........................................................................42

CHAPTER 1

INTRODUCTION

1.1 Introduction

Number Plate Recognition (NPR) is an image-processing technology used to identify vehicles by their number plates. This technology is gaining popularity in security and traffic installations. Much research has already been done for the recognition of number plates. This thesis presents a number plate recognition system as an application of computer vision. Computer vision is a process of using a computer to extract high level information from a digital image.

NPR systems normally consist of an image database, illumination, frame grabber, computer, software and hardware.

1.2 A Typical NPR System

When the vehicle approaches the secured area, the NPR unit senses the car and activates the illumination. The NPR unit takes the pictures from the front plates from the NPR camera. The image of the vehicle contains the number plate. The NPR unit feeds the input image to the system. The system then enhances the image, detects the plate position, extracts the plate, segments the characters on the plate and recognizes the segmented characters.

After that recognized character is send to LCD, after displaying in LCD signals to open the gate. After passing the gate closed. Now the system waits for the next vehicle to approach the secured area.

1.3 Structure of the Proposed NPR System

The system presented is designed to recognize number plates from the front of the vehicle. Input to the system is an image sequence acquired by a digital camera that consists of a number plate and its output is the recognition of characters on the number plate. The system consists of the standard four main modules in an NPR system, viz. Image acquisition, number plate extraction, and number plate segmentation and number plate recognition. The first task acquires the image.

The second task extracts the region that contains the number plate. The third task isolates the characters, letters and numerals, as in the case of Indian Number Plates. The last task identifies or recognizes the segmented characters.

1.3.1 Image Acquisition This is the first phase in an NPR system. This phase deals with acquiring an image by an acquisition method. In our proposed system, we use a high resolution digital camera to acquire the input image. The input image is 3648 x 2056 pixels.

1.3.2 Number Plate Extraction Number Plate Extraction is a key step in an NPR system, which influences the accuracy of the system significantly. This phase extracts the region of interest, i.e., the license plate, from the acquired image. The proposed approach involves “defined region of plate”.

1.3.3 Number Plate Segmentation Number Plate Segmentation, which is sometimes referred to as Character Isolation takes the region of interest and attempts to divide it into individual characters.

1.3.4 Number Plate Recognition The last phase in NPR system is to recognize the isolated characters. After splitting the extracted number plate into individual character images, the character in each image can be identified. There is method used to recognize isolated characters Probabilistic Neural Networks.

1.4 Image processing

1.4.1 Image Pre-processing

The only inputs to the system were the images of vehicles captured by a digital camera. The captured images were taken from approximately 3-5 meters away from the front of the vehicle so that the number plates were clearly visible in the view. Since there are pattern recognition problems arising due to poor image quality caused by varying ambient lighting conditions, number plates are often difficult to detect accurately and efficiently in real situations. Based on the pilot tests carried out with several test runs, for fixed camera, we must maintain car distance constant from fixed camera.

1.4.2 Image Resize

Interpolation is basic tool used extensively in task such as zooming, shrinking, rotating and zoometric correction. Interpolation mainly using here for image resize (zooming and shrinking), which is basically image re-sampling methods. Interpolation is the process of using known data to estimate values at unknown locations.

Different types of interpolation methods

- Nearest neighbour interpolation

It is called nearest neighbor interpolation because it assigns to each new location the intensity of its   nearest neighbor in the original image.

- Bilinear interpolation

Bilinear interpolation is in which we use the four nearest neighbors to estimate the intensity at a given location.

- Bi-cubic interpolation

Bi-cubic interpolation is in which it involves the sixteen nearest neighbors of a point. Thus for doing image resize bi-cubic interpolation method is used.

1.4.3 Image Cropping

Cropping the number plate from car image .by trial error method taking some approximate pixels for all four co-ordinate values where number plate located using that pixel.

1.4.4   Color Image Processing

Color is a power full descriptor that often simplifies object identification and extraction from scene. And humans can discern thousands of color shades and intensities, compared to about only two dozen shades of gray. Second factor is particularly important in manual image analysis. Basically, the colors that humans and some other animals perceive in an object are determined by the nature of the light reflected from object. Visible light is composed of a relatively narrow band of frequencies in the electromagnetic spectrum. Cones are the sensors in the responsible for color vision. The 6 to 7 million cones in the human eye can divided in to three principal sensing categories, corresponding roughly to red, green and blue.

Approximately 65% of all cones  are sensitive to red light,33% cones are sensitive to green light and 2% cones  are sensitive to blue light ,due to  this colours are seen as variable combination of the so called “primary colours red(R),green(G),and blue(B).

An RGB image, sometimes referred to as a true color image, is stored as an m-by-n-by-3 data array that defines red, green, and blue color components for each individual pixel. The color of each pixel is determined by the combination of the red, green, and blue intensities stored in each color plane at the pixel's location. Graphics file formats store RGB images as 24-bit images, where the red, green, and blue components are 8 bits each. This yields a potential of 16 million colors. The precision with which a real-life image can be replicated has led to the nickname "true color image".

M-by-n-by-3 array of class uint8, uint16, single, or double whose pixel values specify intensity values. For single or double arrays, values range from [0, 1]. For uint8, values range from [0, 255]. For uint16, values range from [0, 65535].

Gray scale =0.2989 * R + 0.5870 * G + 0.1140 * B

1.4.5 Gray Scale Images

A grayscale image (also called gray-scale, gray scale, or gray-level) is a data matrix whose values represent intensities within some range. MATLAB stores a grayscale image as a individual matrix, with each element of the matrix corresponding to one image pixel.

Array of class uint8, uint16, int16, single, or double whose pixel values specify intensity values. For single or double arrays, values range from [0, 1]. For uint8, values range from [0,255]. For uint16, values range from [0, 65535]. For int16, values range from [-32768, 32767].

Graythresh(I) computes a global threshold (level) that can be used to convert an intensity image to a binary image with im2bw. level is a normalized intensity value that lies in the range [0, 1].The graythresh function uses Otsu's method, which chooses the threshold to minimize the intraclass variance of the black and white pixels.

Im2bw (I, level) converts the grayscale image I to a binary image. The output image BW replaces all pixels in the input image with luminance greater than level with the value 1 (white) and replaces all other pixels with the value 0 (black). You specify level in the range [0, 1], regardless of the class of the input image.

1.4.6   Binary Images

In a binary image, each pixel assumes one of only two discrete values: 1 or 0. A binary image is stored as a logical array. Binary images are easily to process further processing like removing noise, filtering, segmentation, recognition etc.

1.4.7 Intensity Transformation

Intensity transformation operates on single pixels of an image, principally for the purpose of contrast manipulation and image threshold. Intensity transformation is among the simplest of all image processing techniques. The values of pixels, before and after processing, will be denoted by r and s respectively. s=T(r), where T is a transformation that maps a pixel value r in to a pixel values.

Three basic Intensity Transformations:

- Linear (negative and identity transformation).

The negative of an image with intensity levels in the range [0, L-1] is obtained by using the negative transformation

s=L-1-r

Reversing the intensity levels of an image in this manner produces the equivalent of a photographic negative. By binary image we can use this formula reversing the image

s=1-r

- Logarithmic (log and inverse log transformation).

The general form of the log transformation

s=c log (1+r)

Where c is a constant, and it is assumed that r ≥ 0  .The narrow range of low intensity values in put in to  a wider range of output levels and higher range values become narrow. We use a transformation of this type to expand the values of dark pixels in an image while compressing the higher –level values. The opposite is true of the inverse log transformation.

- Power –law (nth power and nth   root transformation).

Power-Law transformation has the basic form

s=crγ

where c and γ are positive constants.

Plot of s and r for various values of γ. power law curves with fractional values of γ map a narrow range of dark input values in wider range of output values, with opposite being true for higher values of input levels the curves generated with values of γ>1 have exactly the opposite effect as those generated with values of γ<1.c= γ=1 we getting identity transformation. The identification of objects within an image can be a very difficult task. One way to simplify the problem is to change the gray scale image into a binary image, in which each pixel is restricted to a value of either 0 or 1. The techniques used on these binary images go by such names as: blob analysis, connectivity analysis, and morphological image processing. The foundation of morphological processing is in the mathematically rigorous field of set theory; however, this level of sophistication is seldom needed. Most morphological algorithms are simple logic operation.

### 1.4.8 Pixel Connectivity

### Morphological processing starts at the peaks in the marker image and spread throughout the rest of the image based on the connectivity of the pixels. Connectivity defines which pixels are connected to other pixels. A set of pixels in a binary image that form a connected group is called an object or a connected component.

Estimate the area of all of the pixels in an image by summing the areas of each pixel in the image. The area of an individual pixel is determined by looking at its 2-by-2 neighborhood. There are six different patterns, each representing a different area:

- Patterns with zero on pixels (area = 0)

- Patterns with one on pixel (area = 1/4)

- Patterns with two adjacent on pixels (area = 1/2)

- Patterns with two diagonal on pixels (area = 3/4)

- Patterns with three on pixels (area = 7/8)

- Patterns with all four on pixels (area = 1)

Each pixel is part of four different 2-by-2 neighborhoods. This means, for example, that a single on pixel surrounded by off pixels has a total area of 1.

1.5 Neural Network Introduction

Neural networks are composed of simple elements operating in parallel. These elements are inspired by biological nervous systems. As in nature, the connections between elements largely determine the network function.  Train a neural network to perform a particular function by adjusting the values of the connections (weights) between elements. Typically, neural networks are adjusted, or trained, so that a particular input leads to a specific target output. There, the network is adjusted, based on a comparison of the output and the target, until the network output matches the target. Typically, many such input/target pairs are needed to train a network. Neural networks have been trained to perform complex functions in various fields, including pattern recognition, identification, classification, and speech, vision, and control systems. The  figure 1.1 shows the neural network training diagram.

Neural   Network

Including Connections

(Called Weights)

Between neurons

Input

Adjust   Weights

Target

Compare

Output

Fig 1.1 Neural Network Training Diagram

1.6 Microcontroller Introduction

A micro-controller can be compared to a small stand alone computer, it is a very powerful device, which is capable of executing a series of pre-programmed tasks and interacting with other hardware devices. Being packed in a tiny integrated circuit (IC) whose size and weight is usually negligible, it is becoming the perfect controller for robots or any machines requiring some kind of intelligent automation.

A single microcontroller can be sufficient to control a small mobile robot, an automatic washer machine or a security system. Any microcontroller contains a memory to store the program to be executed, and a number of input/output lines that can be used to interact with other devices, like reading the state of a sensor or controlling a motor.

Nowadays, microcontrollers are so cheap and easily available that it is common to use them instead of simple logic circuits like counters for the sole purpose of gaining some design flexibility and saving some space. Some machines and robots will even rely on a multitude of microcontrollers, each one dedicated to a certain task. Most recent microcontrollers are 'In System Programmable', meaning that we can modify the program being executed, without removing the microcontroller from its place.

1.7 Max 232

Serial RS-232 communication works with voltages “1” represent   -3 to -25 and   “0” represent +3 to +25, making -3 to 3 undefined. For this reason .to connect any RS-232 to microcontroller system. MAX 232 to convert the TTL logic level to the RS 232 voltage level, and vice versa. MAX232 IC chips are commonly referred to as line drivers.

1.8 LCD Display

LCD or liquid Crystal Display has different types of LCD available. In this project 16x2 lines and 5x7 matrixes is used. LCD contains logic analyzer to controlling display and also communication with microcontroller. It contains 16 pins for different purpose like data transmission controlling display.

1.9 Introduction ULN 2803

Since the digital outputs of the some circuits cannot sink much current, they are not capable of driving relays directly. So, high-voltage high-current Darlington arrays are designed for interfacing low-level logic circuitry and multiple peripheral power loads. The series ULN2803 ICs can drive 8  stepper motor At an appropriate duty cycle depending on ambient temperature and number of drivers turned ON simultaneously, typical power loads total over 260W [400mA x 7, 95V] can be controlled. Typical loads include relays, solenoids, stepping motors, magnetic print hammers, multiplexed LED and incandescent displays, and heaters. These Darlington arrays are furnished in 16-pin dual in-line plastic packages (suffix A) and 16-lead surface-mountable SOICs (suffix L). All devices are pinned with outputs opposite inputs to facilitate ease of circuit board layout.

1.10 Stepper Motor

Of all motors, step motor is the easiest to control. Its handling simplicity is really hard to deny - all there is to do is to bring the sequence of rectangle impulses to one input of step controller and direction information to another input. Direction information is very simple and comes down to "left" for logical one on that pin and "right" for logical zero. Motor control is also very simple - every impulse makes the motor operating for one step and if there is no impulse the motor won't start.

Pause between impulses can be shorter or longer and it defines revolution rate. This rate cannot be infinite because the motor won't be able to "catch up" with all the impulses.

The key to driving a stepper is realizing how the motor is constructed. A diagram shows the representation of a 4 coil motor, so named because 4 coils are used to cause the revolution of the drive shaft. Each coil must be energized in the correct order for the motor to spin. Responsible for providing the control signals to open and close the switches at the appropriate times in order to spin the motors. The control unit is commonly a computer or programmable interface controller, with software directly generating the outputs needed to control the switches. As with drive circuitry for variable reluctance motors, the inductive kick produced when each of these switches is turned off.

Step angle

It is angle through which motor shaft rotates in one step. The step angle is different for different motors. Selection of motor according to step angle depends on the application, simply if we require small increments in rotation choose motor having smaller step angle.

No of steps require to rotate one complete rotation = 360 deg. / step angle in deg.

Steps/second

The relation between RPM and steps per sec. is given by,

Steps or impulses /sec. = (RPM X Steps /revolution) /60

Pause between impulses can be shorter or longer and it defines revolution rate. This rate cannot be infinite because the motor won't be able to "catch up" with all the impulses (documentation on specific motor should contain such information). So referring to RPM value in datasheet we can calculate steps/sec and from it delay or pause between impulses.

1.11 Problem Statement

Software implementation of character recognition in vehicle number plates by using Neural Network and Development of hardware and software for character display in LCD and control of stepper using microcontroller as well as Testing of the hardware and software developed for the system.

1.12 Objective

The work presented here aims at the following aspects.

- Study of MATLAB Toolbox like Image Processing and Neural Network toolbox.

- Study the existing license plate recognition systems.

- Develop a car license plate recognition system using Image Processing Toolbox and Neural Network Toolbox.

- Integrate between Image Processing and Neural Network.

- Integrating MATLAB and microcontroller 8051.

- Compare the various techniques at hand with the proposed system.

CHAPTER 2

LITERATURE SURVEY

Number plate recognition systems have received a lot of attention from the research community. Much research has been done on Korean, Chinese, Dutch and English number plates. A distinctive feature of research work in this area is being restricted to a specific region, city, or country. This is due to the lack of standardization among different number plates (i.e., the dimension and the layout of the license plates).

Any basic NPR system has 3 basic problems,

- Number plate detection

- Character segmentation

- Character recognition

2.1 Number Plate Detection Approaches

Image detection deals with the location of the Number plate on the vehicle, various types of image processing techniques have been used for this. Image detection becomes easier if a standardized location for the placement of number plate is defined for all vehicles. Edge detection techniques[1], bounder line detection techniques are some of the techniques that can used to locate the number plate here the image is converted into a grey scale image and the intensity is checked along the row and column, the region which has high intensity has the license plate information. Other techniques that can be used are the Top hat Bottom hat filtering where the black to white transitions are highlighted and Binary Morphological algorithms can also be used but these approaches rely on color information and special signs.

2.2 Character Segmentation Approaches

There are many common algorithms for Character Segmentation  such  direct segmentation  [2],  projection  and  cluster  analysis  [3]  and  template  matching  [4].  In direct segmentation characters are segmented directly based on the width of the characters and spaces between them. This algorithm is fast but the boundaries of the character region must be located accurately. In Projection and Cluster Analysis the number of white pixels in each vertical column is counted and recorded. The character regions have more white pixels as compared to the region between them.

By detecting  the  trough  of  white  pixels  between  characters  the  spaces  can  be located. In  template  matching  algorithm,  a  template  is  used  to  scan  the  image  and  find  the maximum difference  value  of  white  pixels  between  the  region  of  characters and  the  region between the characters. Other approaches for segmentation includes segmentation through region growing approach and the most common one is segmentation using vertical and horizontal image histogram  methods.  The  region  growing  method  takes  quite  much  of  the  computation  time  so histogram method is mostly used which is very fast and its accuracy is also considerable.

2.3 Character Recognition Approaches

Various approaches have been defined for the purpose of character recognition and identification and most of the approaches do it individually for each identified character. Some of the previous work in the classification and recognition of characters is as follows, a statistical pattern recognition[5] approach for recognition was used but the technique was found to be inefficient.  This  approach  is  based  on  the  probabilistic  model  and  uses statistical  pattern  recognition  approach. One of the approach  identifies  the  characters based on the  number  of  black  pixel  rows  and  columns  of  the  character  and  comparison  of  those values to a set of templates[6]  or signatures in  the database.

Template matching[7] involves the use of a database of characters or templates.  There is a separate template for each possible input character.  Recognition is achieved by comparing the current input character to each of template in order to find the one which matches the best. Another technique adopted a structural or syntactic approach[8] to recognize characters in a text document; this technique can yield a better result when applied on the recognition of individual characters. This approach is based on the detection of holes and concavities[9] in the four directions (up, down, left and right), which permits the classification of characters into different classes. In addition, secondary characteristics are used in order to differentiate between the characters of each class. The  approaches  discussed here are  based  on  the  structural  information  of  the  characters  and  uses  syntactic pattern recognition approach.  Neural network[10] can be used for the recognition phase.  Neural network accepts any set of distinguishable set of characters as input.  It  then  trains  the  network  using  the  input  data  and  the training algorithms to recognize the input pattern (In this case characters).

2.4 Challenges in LPR System

The attributes of the License plates play important role in the recognition process. If the size, color and the character height and width[11] is pre-defined then the recognition process will be much easier. There are several countries in the world who have adapted this very method of standardizing the License plate. But in India, the license plate is purely localized and even they have different scripts for the characters. So, the recognition process is quite difficult. Other  than  this,  there  are  several  other  parameters  on  which  the  quality  of  the  recognition depends.  Mostly  all  the  available LPR  systems  have  certain  constraints  on  them  and  they benefit from these controlled scenarios.

These constraints are

- Lack of background objects: The image captured by the camera doesn’t have much background details.

- High quality imaging: The quality of the captured image is high, so that it will be easy to evaluate the attributes of the image.

- Minimal skewing and rotation: The camera is fit such that the captured image will not suffer much decent angle of skewing and rotation.

- Better Flash: As the License plate is retro- reflective in nature, so if high illumination will be there, then the LP region will be much more brightened and the attributes of the license plate will not be much clearer.

Apart from these conditions, the algorithms applied for the recognition[12] also plays a vital role. If the quality of the algorithm is good, then more varieties of images can be input to the system. And this will reduce the computation speed of the process also. The most basic issues in the real-time LPR system is the accuracy and the recognition speed.

If we want to have higher accuracy then, the system will take much time. But  if  we  are  working  in  a  controlled  scenario,  then  we  can  compromise  with  the  accuracy because  the  quality  of  the  captured  image  is  good.  This way we can reduce the computation time. Some problematic captured images of the license plate is shown in figure 2.1.

Fig 2.1 Problematic Captured Images of License Plate

2.5 Commercial Products

The various products in the market today are described briefly below.

2.5.1 IMPS (Integrated Multi-Pass System)

An IMP is a Singaporean commercially developed license plate recognition system. It is a high performing robust system that gives consistent results under all weather conditions. Using advanced image processing and artificial intelligent  techniques such as  AI best first breadth-wise search algorithm[13], combined template and neural network recognizers, fuzzy logic and an arsenal of image processing tools, it automatically locates vehicle license plates and reads the numbers accurately each time every time.

2.5.2 Perceptics

Perceptics is the world leader in license plate reader technology. Current LPR system read Latin (A-Z) and Korean (Hangul) letter and Arabic number (0-9); however, the LPR can be  programmed  to  read  any  language  or  symbol  in  any  alphanumeric  combination  or context  on  both  retro  and  non-retro[14]  reflective  plates.

With milliseconds the LPR system locates, captures and identifies a vehicle's license plate data and makes a read decision. The system's reliability and flexibility allow it to accommodate some of the most stringent needs in some of the worst conditions.

Features of this LPR technology includes,

- Automatic and within milliseconds.

- Reads accurately in most weather conditions.

- Reads accurately at highway speeds.

- Works 24 hours a day, 7 days a week.

2.5.3 Vehicle Identification System for Parking Areas (VISPA)

PPI's Vehicle Identification System for Parking Areas (VISPA) uses video imaging for better recognition, identification and improved security. VISPA provides for state-of-the-art video  technology,  easy  installation  and  has  accessories  and  features  for  most  parking security surveillance needs.

Features are,

- Open architecture to most common video-systems.

- Compatible with standard hardware and software.

- Can be customized according to specific user needs.

VISPA is available in two forms:

Basic Version: An image of the car and/or the driver (depending on the location of your camera) will be taken as soon as the car approaches the triggering device. The image will be linked to the ticket. The basic system version connects to 4 cameras and can be upgraded to 8 cameras.

Enhanced Version: License Plate Identification, The VISPA controller with an integrated frame grabber card for 4, 8, or 16 cameras automatically identifies the license plate from the video image and stores it in a database. The license plate can then be encoded on the ticket.

2.5.4 Hi-Tech Solution

Hi-Tech Solutions  is a system and software company that develops cutting edge optical character  recognition[15]  solutions  by  implementing  the  company's  unique  image processing  software  and  hardware  in  a  wide  range  of  security  and  transportation applications.

Their  technology  is  based  on  computer  vision,  the  system  read  the  camera images and extract the identification  data  from the  images. The recognition result is then logged together with the images.

This is the main advantage of vision based recognition; the records include both the image plus the extracted result. Their product includes,

See Car License Plate Recognition: Detects and reads Vehicle license plates for parking, access control, traffic surveillance, law enforcement and security applications. Available as a complete system which is based on a background Windows application, Windows DLL or Linux library, as a stand-alone turn-key version, or in form of different special-task systems.

See Container Identification System: Tracks and reads Shipping container identification marking,  and  transmits  the  ID  string  to  the  port  or  gate  computer,  or  to  a  client  process. Available as complete systems, such as SeeGate which is a recognition system for the Tracks and Containers, or SeeCrane which is a crane mounted Container recognition system.

CHAPTER 3

MODEL OR BLOCK DIAGRAM

3.1 Project Block Diagram

The image captured is stored in database. The image read from the database is converted to standard size; noise present in the image is removed. Then the preprocessed image is given as input to segmentation. Then find the height and width of each character of the image. After this it is given as input to neural network, the character recognized is sent as input to microcontroller. The Microcontroller receives data from the PC, and sends it to LCD, for displaying. After the Character is displayed on LCD, Microcontroller rotates stepper motor 90 degrees clock wise and anticlockwise direction. The project block diagram is as shown in figure 3.1

TRAIN BY

NEURAL NETWORK

NUMBER PLATE IMAGE AS THE INPUT

COMPUTER PROCESSING USING MATLAB OUTPUT           KA 04 ME 6152

COMPARE WITH IMAGE DATABASE TO AUTHENTICATEU

MUC

8051

KA 04 ME 6152

LCD NUMBER PLATE DISPLAY

ULN

2803

STEPPER MOTOR ROTATES TO OPEN TOLLGATE

Fig 3.1 Project Block Diagram

The explanation for individual parts is given below:

- Number Plate Image

The image of the number plate to be identified is given as input to the computer for processing. The image is taken by a high resolution camera of around 10.1 Mega pixels. The image may be in the form of .jpg, .png, .gif, .jpge, .bmp which is to be processed by the computer.

- Computer Processing By Matlab

The image given as input is processed by matlab software for various fundamental image processing steps that includes acquisition which accepts the high resolution input image and converts it into matrix form. Image pre processing helps us to improve the image in ways that increase the chances for success of the other processes.

Image segmentation helps to partitions an input image into its constituent parts or objects. Image representation helps to convert the input data to a form suitable for computer processing. Image description helps us to extract features that result in some quantitative information of interest or features that are basic for differentiating one class of objects from another. Image recognition helps us to assign a label to an object based on the information provided by its descriptors. Image interpretation helps us to assign meaning to an ensemble of recognized objects.

- Training Images By Neural Network

The recognized characters in the number plate are stored as separate images by considering 90 x 40 images. The characters are separated and the separated characters are arranged in their respective databases, for e.g. all k’s are grouped in a separate ‘K’ database. Thereafter the neural network code is trained so as to recognize those characters in the real world applications.

- Compare With Image Database To Authenticate

The number plate recognized has ten characters which are compared with number plate in database and only those in the database are authenticated.

- Microcontroller 8051

The 8051 is used to interface stepper motor and lcd for display and tollgate operation. It is also used for serial communication interface between the system and 8051 via max232.

- LCD Display

The lcd is used to display the number plate and also whether the number plate is “valid” or “not valid” and accordingly the gate opened message is displayed.

- ULN 2803

This is the driver used to run the stepper motor by interfacing it with 8051 and the stepper motor.

- Stepper Motor

The stepper motor is rotated by 90 degrees clockwise to open the toll gate when the number plate is valid.

3.2 Block Diagram for Microcontroller Section

Designing of Microcontroller part is shown with the connecting diagram. Microcontroller used is Atmel 8051. It is a 40 pin IC. It requires 5V power supply. Port 0 is connected to LCD of DB0 – DB7 and in pin P1.0 is connected to pin no. 4 of LCD display. Pin P1.1 is connected to pin no. 5 of LCD display and similarly in pin P1.2 is connected to pin no. 6 of LCD display. Microcontroller port P2 is is connected to ULN 2803 from pin no.1 to pin no.8. Microcontroller pin No. 10 and 11 are connected to MAX 232 with pin no.12 and 11 respectively. Microcontroller pin no. 18 and 19 is connected to Crystal oscillator with a frequency of 11.0952MHz.  Pin no.31 of microcontroller is connected to VCC because internal Flash memory is used. Max 232 converts TTL to RS232 level and ULN 2803 using for driver for Stepper Motor.

### Pin   Description

- PINS 1-8 (PORT 1):- Each of these pins can be configured as an input or an output.

- PIN 9: RS A logic one on this pin disables the microcontroller and clears the contents of most registers. In other words, the positive voltage on this pin resets the microcontroller. By applying logic zero to this pin, the program starts execution from the beginning.

- PINS 10-17 (PORT 3):- Each of these pins can serve as general input or output. Besides, all of them have alternative functions:

- PIN 10: RXD Serial asynchronous communication input or Serial synchronous communication output.

- PIN 11: TXD Serial asynchronous communication output or Serial synchronous communication clock output.

- PIN 12: INTO Interrupt 0 input.

- PIN 13: INT1 Interrupt 1 input.

- PIN 14:T0 Counter 0 clock input.

- PIN 15: T1 Counter 1 clock input.

- PIN 16: WR Write to external (additional) RAM.

- PIN 17: RD Read from external RAM.

- PIN 18 & 19 (X2, X1):- The 8051 requires the existence of an external oscillator circuit. The oscillator circuit usually runs around 12MHz, although the 8051 (depending on which specific model) is capable of running at a maximum of 40MHz.

- PIN 20: Ground.

- PIN 21-28(PORT 2):- If there is no intention to use external memory then these port pins are configured as general inputs/outputs. In case external memory is used, the higher address byte, i.e. addresses A8-A15 will appear on this port. Even though memory with capacity of 64Kb is not used, which means that not all eight port bits are used for its addressing, the rest of them are not available as inputs/outputs.

- PIN 29 : PSEN  "Program Select Enable" If external ROM is used for storing program then a logic zero (0) appears on it every time the microcontroller reads a byte from memory.

- PIN 30: ALE “Address Latch Enable” Which is used when multiple memory chips are connected to the controller and only one of them needs to be selected.

- PIN 31: EA /VPP  By applying logic zero to this pin, P2 and P3 are used for data and address transmission with no regard to whether there is internal memory or not. It means that even there is a program written to the microcontroller, it will not be executed. Instead, the program written to external ROM will be executed. By applying logic one to the EA pin, the microcontroller will use both memories, first internal then external (if exists).

- PIN 32-39:- PORT 0 Similar to P2, if external memory is not used, these pins can be used as general inputs/outputs. Otherwise, P0 is configured as address output (A0-A7) when the ALE pin is driven high (1) or as data output (Data Bus) when the ALE pin is driven low (0).

- PIN 40: VCC +5V power supply.

Figure 3.2 shows the microcontroller connection diagram.

Figure 3.2 Microcontroller Connection Diagram

CHAPTER 4

DESIGN AND IMPLEMENTATION

4.1 Flow Chart for Image Processing Section

In this image processing flow steps the fundamental processes of image processing like image acquisition and morphological processing are used to perform the number plate extraction.

Start

Image capturing

Images   Stored in database

Images read from data base

RGB to gray conversion

Image cropping

Image resize

A

Gray to binary conversion

Pre –processed image

Inversing of Image

Inverse of Image

Getting ROI part

Removing noise

Image filling

Finding the width and height if each character

Extracting region of objects

Resizing of extracted images

Converting images to column vector then sending I/P Neural Network

A

B

If image is identified?

Respective character identified and stored in variable result

It is consider blank

Stepper motor rotating 90° clock wise and 90° anti clock wise

Sending recognized characters serially to microcontroller

Microcontroller received characters from PC displaying In LCD

Yes

No

B

Stop

Fig 4.1 Flow chart for Image Processing Section

The individual characters are then extracted using the segmentation process in which the position of each character pixel wise in rows and columns is stored in some variable. The variables are then used to extract the character by cropping it from the main image and creating a sub image.  The sub images of characters are then trained by artificial neural networks for the process of authentication. The figure 4.1 shows the flow chart for image processing section.

4.2 Program Flow Chart for Microcontroller Section

Initialization of serial communication, baud rate, data bits, start and stop bits will be done at the beginning. Initialize LCD, then display message on LCD.

After the characters received from PC, the received characters are displayed on LCD. Then the Microcontroller controls the stepper motor clockwise and anticlockwise direction. The flow chart for microcontroller section is as shown in figure 4.2.

Start

Enable the interrupt serial, into and int1 using IE register.IE=#95H

Initialize of TMOD register Timer 1, mode 2

Set the timer 1

Selecting the data, stop bit using SCON register

Initialize the LCD

X

x

Stepper motor rotates clock wise direction 90° and displays In LCD “GATE OPENED”.

Stepper motor rotates anti clock wise direction 90° and displays In LCD “GATE CLOSED”.

Clear LCD screen

Clear LCD screen and waiting from next data from PC

End

Writing In LCD “WEL COME TO NUMBER PLATE”   message

Clear LCD screen

Waiting for receiving characters from personal computer

If first character received from personal computer displaying first line in LCD “NUMBER PLATE” and second  line character received from PC

Fig 4.2 Flow chart for Microcontroller Section

4.3 Project Implementation Steps

4.3.1 Implementation Steps for Image Processing Section

Step 1

Captured RGB image stored in data base. With size 3648 columns and 2057 rows

Captured Image

Step 2

Convert RGB image to Gray scale image using function rgb2gray.

RGB to Gray scale image

Step 3

Resize the “gray scale” image using function imresize.

Image resized 1000Rand 2000C

2000

Step 4

Crop the number plate location image 601 rows and 1701 columns.

Cropped Image 601R and 1701C

Step 5

Convert the gray scale image to binary image using function “im2bw”.

Gray to binary image

Step 6

Image fill by holes, it fills the surround by back ground white pixels using function “imfill”.

Image fill

Step 7

Inversed image from step 6

Inverse image

Step 8

Adding the images in step 5 and step 7 to get the final image.

Step 9

Now inversed the image from step 7 is taken now object become white and background become black.

Inversed image

Step 10

Remove the small connected white pixels using function “bwareaopen”.

Removed small noise

Step 11

Finally noise removed image, this input image to segmentation.

Pre-processed image

Step 12

Adding the each column individually, add value available in “projh”.

Sum individually each column

Step 13

Adding the each row individually, add value available in “projv”.

Sum individually each row

Step 14

Find some threshold value.

Step 15

Plot the vertical and horizontal graphs using threshold value.

Plotting using threshold

Plotting using threshold

Fig 4.3 Implementation Steps for Image Processing Section

The steps 1-15 are the implementation steps for image processing section which is as shown in figure 4.3.

4.3.2 Finding Width and Height of each character in the Image

Step 16

Register the portions where values change from 0 to 1 and from 1 to 0 in xloc (d) are respectively.

xloc

Columns 1 through 12

297         377         394       471         539         622           631         702         759            851         863         942

Columns 13 through 21

1000        1083        1114      1151        1162        1246        1254        1335        1701

Xloc

Columns 1 through 12

297         377         394       471         539         622           631         702         759            851         863         942

Columns 13 through 20

1000        1083        1114      1151        1162        1246        1254        1335

Step 17

Find the height of each character.

yloc1

166   280

165   279

165   277

165   277

162   275

161   273

160   270

156   268

155   268

154   266

Step 18

Dividing height by width if height by width less <18 it indicates candidate is there, then extracting that region.

Extracted Objects

Step 19

Resize all the candidates with 90 rows and 40 columns.

Resized Images

Fig 4.4 Extracted Objects and Resized Images

Step 20

Converting 90x40 matrix into row vector.

The steps 16-20 are carried out to extract objects and resize images. The figure 4.4 shows the extracted objects and resized images.

4.3.3 Implementation Steps for Character Recognition using Neural Network

Step 21

Row is converted in to column giving input to neural network.

Step 22

Step 18 and 19 repeat at the end of candidate.

Step 23

Recognized characters located variable result and str

Step 24

Send each character serially at the baud rate of 9600.

Step 25

Display result in notepad using function “winopen”.

Fig 4.5 Character Display

The character display is as shown in figure 4.5.

4.3.4 Recognized Character Displaying in LCD and controlling Stepper Motor using Microcontroller

Step 26

Microcontroller receives all character from computer at the baud rate of 9600.

Step 27

Receive all character display in LCD.

Step 28

After receiving all the character stepper motor rotates 90° clock wise direction and then 90°   anti clock wise direction.

4.4 Microcontroller Data Flow Steps

4.4.1 Initialization

Step 1

Enable the interrupt serial, into and int1 using IE register.

Step 2

Select timer 1 mode 2 for serial communication using TMOD register.

Step 3

Calculating baud rate using following formula and loaded in to TH1 register.

XTAL=11.0592

Baud rate=11.0592 MHZ/12=921.6 KHZ/32=28,800HZ/3=9600

-3 or FD(hex) is loaded in to TH1

Step 4

Select of data bits, start and stop bit using SCON register.

Step 5

Set the timer TR1 register.

Step 6

By sending the command to initialize the LCD with 16x2, 5X7 matrix and the cursor as ON to LCD display.

4.4.2 Displaying Message on LCD

Step 7

Writing data in to the LCD “WEL COME TO NUMBER PLATE”

Step 8

Clearing the LCD screen by sending the command #01H.

Step 9

Waiting for characters receive from Personal Computer (PC).

Step 10

After receiving the first character from PC, display first line of LCD “NUMBER PLATE”. Then in second line displaying all characters receive from PC (Recognized characters).

Step 11

Clear the screen of LCD.

4.4.3 Controlling Stepper Motor

Step 12

Stepper motor rotates clock wise direction 90° and displays In LCD “GATE OPENED”.

Step 13

Stepper motor rotates anti clock wise direction 90° and displaying In LCD “GATE CLOSED”.

Step 14

Clear the screen of LCD.

CHAPTER 5

RESULTS AND CONCLUSION

5.1 Settings

Due to the limited availability of data at the time of experiment, the NPR system was tested by using a reduced set of data. A total of 17 images with vehicle license plates taken under different lighting conditions, different distance and different exposure were used as input to the system. The images were originally taken using a 10.1Mega-pixel digital camera and resized to 1000x2000 for quicker processing.

5.2 Result

5.2.1. Plate Localization

All images were submitted to the plate localization process, candidate regions are marked with a red rectangle after processing. The images are visually inspected to verify its correctness.

Plate Localization Success Rate

| Total Images | Plate located | Failed to locate | Success Rate |
| --- | --- | --- | --- |
| 17 | 17 | 0 | 100 |

Table 5.1 Plate Localization

5.2.2 Character Segmentation

Only those images with candidate regions identified are submitted to Character Segmentation. After processing, the plate area is extracted from the input image.

Character Segmentation Success Rate

| Images Character | Character Segmented | Failed | Success rate |
| --- | --- | --- | --- |
| 17 | 17 | 0 | 100 |

Table 5.2 Character Segmentation

The table 5.1 shows the plate localization success rate and table 5.2 shows the character segmentation success rate.

5.2.3 Character Recognition

It recognizes all trained sample but it may be recognize other than trained samples. For this the training of probabilistic Neural Network is necessary.

5.3 Conclusion

The process of vehicle number plate recognition requires a very high degree of accuracy when we are working on a very busy road or parking which may not be possible manually as a human being tends to get fatigued due to monotonous nature of the job and they cannot keep track of the vehicles when there are multiple vehicles are passing in a very short time .To overcome this problem, many efforts have been made by the researchers across the globe for last many years. A similar effort has been made in this work to develop an accurate and automatic number plate recognition system. MATLAB has been used to obtain the desired results. The setup has been tested for 17 vehicles containing different number plates. In the process of final evaluation after optimizing the parameters like brightness, contrast and gamma, adjustments, optimum values for lightening and the angle from which the image is to be taken. We get an overall efficiency of 100% for this system. Though this accuracy is not acceptable in general, but still the system can be used for vehicle identification. It can give us a relative advantage of data acquisition and online warning in case of stolen vehicles which is not possible by traditional man handled check posts.

The Neural Network trained with 17 vehicle boards for character recognition. The same 17 vehicle boards have been used for testing the software. It recognized all the boards. However other boards which are not trained could not be recognized, because capturing image different angle, distance and also size of fonts different in number plates. Some of the number plates have not recognized due to localization and segmentation errors.

5.4 Future Work

From this project it has been clear that it is easy to implement Character Recognition in number of applications. This project is detecting particular number plate position. Further designing an algorithm for detecting number plate in any position of the vehicle is to be done. In this project it is recognizing trained samples, further designing an algorithm for recognition with less number of samples and capturing the number plates with different angles, algorithm has to be developed. So that it gives more accurate and reliable.

CHAPTER 6

APPLICATIONS OF NPR SYSTEM

Vehicle number plate recognition is one form of automatic vehicle identification system. NPR systems are of considerable interest, because of their potential applications to areas such as highway electronic toll collection, automatic parking attendant, petrol station forecourt surveillance, speed limit enforcement, security, customer identification enabling personalized services, etc. Real time NPR plays a major role in automatic monitoring of traffic rules and maintaining law enforcement on public roads.

Law Enforcement: The plate number is used to produce a violation fine on speeding vehicles, illegal use of bus lanes, and detection of stolen or wanted vehicles. The main advantage is that the system can store the image record for future references.

The front part of the vehicle is extracted off the filmed image and is given to the system for processing. The processed result is fed into the database as input. The violators can pay the fine online and can be presented with the image of the car as a proof along with the speeding information.

Parking:  The NPR system is used to automatically enter pre-paid members and calculate parking fee for non-members. The car plate is recognized and stored upon its exit the car plate is read again and the driver is charged for the duration of parking.

Automatic Toll Gates: Manual toll gates require the vehicle to stop and the driver to pay an appropriate tariff. In an automatic system the vehicle would no longer need to stop. As it passes the toll gate, it would be automatically classified in order to calculate the correct tariff.

Border Crossing: This application assists the registry of entry or exits to a country, and can be used to monitor the border crossings. Each vehicle’s information is registered into a central database and can be linked to additional information.

CHAPTER 7

BIBLOGRAPHY

[1] Park, S. FL, Kim, K. I., Jung, K., and Kim, H. J., "Locating car license plates using neural network", IEE Electronics Letters, vol.35, no. 17, pp. 1475-1477, 1999.

[2] Morel J., and Solemini S., "Variational Methods in Image Segmentation", Birkhauser, Boston, 1995.

[3] Nieuwoudt, C, and van Heerden, R., "Automatic number plate segmentation and recognition", Seventh annual South African workshop on Pattern Recognition, pp. 88-93, IAPR, 1996.

[4] Cho, D. U., and Cho, Y. Ft., "Implementation of pre-processing independent of

environment and recognition and template matching ", The Journal of the Korean Institute of Communication Sciences, vol. 23, no. 1, pp. 94-100, 1998.

[5] Hu, M. K., "Visual Pattern Recognition by Moment Invariant", IRE Transaction on Information Theory, volume IT- 8, pp. 179-187, 1962.

[6] Lee, E. R., Earn, P. K., and Kim, H. J., "Automatic recognition of a car license plate      using color image processing", IEEE International Conference on Image Processing 1994, vol. 2, pp.301-305, 1994.

[7] Kim, K. K., Kim, K. I., Kim, J.B., and Kim, H. J., "Learning based approach for license plate recognition", Proceedings of IEEE Processing Society Workshop on Neural Networks for Signal Processing, vol. 2, pp: 614-623, 2000.

[8] Yu, M., and Kim, Y. D., "An approach to Korean license plate recognition based on vertical edge matching", IEEE International Conference on Systems, Man, and Cybernetics, vol. 4, pp. 2975-2980, 2000.

[9] Khotanzad, A., and Hong, Y.H., "Rotation in-variant image recognition using features selected via a systematic method," Pattern Recognition, vol. 23, no. 10, pp. 1089-1101, 1990.

[10] “Adaptive Boosting of Neural Networks for Character Recognition” Holger Schwenk Yoshua Bengio Dept. Informatique et Recherche Op´erationnelle Universit´e de Montr´eal, Montreal, Qc H3C-3J7, Canada fschwenk,bengioyg@iro.umontreal.ca May 29, 1997.

[11] Comelli, P., Ferragina, P., Granieri. M. N., and Stabile, F., "Optical recognition of   motor vehicle license plates", IEEE Transactions on Vehicular Technology, vol. 44, no. 4, pp: 790-799, 1995.

[12] Kim, G. M., "The automatic recognition of the plate of vehicle using the correlation coefficient and Hough transform", Journal of Control, Automation and System Engineering, vol. 3, no.5, pp. 511-519, 1997. 75.

[13] Khotanzad, A., and Hong, Y.H., "Invariant image recognition by zernike moments," IEEE Trans. Pattern Analysis and Machine Intelligence, vol. 12, no. 5, pp. 489-497, 1990.

[14] Naito, T. Tsukada, T. Yamada, K. Kozuka, K. and Yamamoto, S., "Robust recognition methods for inclined license plates under various illumination conditions outdoors", Proceedings IEEE/IEEJ/JSAI International Conference on Intelligent Transport Systems.

[15] Efficient Embedded Neural-Network-Based License Plate Recognition System Hakan Caner, H. Selcuk Gecim, and Ali Ziya Alkar, Member, IEEE.